| Excavation and Digging - PTW Number: |  |
| --- | --- |

Introduction

This Excavation and Digging Control Form establishes mandatory requirements for planning and executing excavation activities at Port of Brisbane sites. Its purpose is to ensure all excavation work is conducted in accordance with the Work Health and Safety Regulation 2011 (QLD) and the Excavation Work Code of Practice 2021, minimizing risks to personnel, infrastructure, and the environment.

The form provides a structured process to:

- Identify and manage underground services before work begins.

- Implement controls to prevent excavation collapse and ensure safe entry.

- Address atmospheric hazards, groundwater, and environmental conditions.

- Document emergency response measures and isolation requirements.

- Verify compliance with site-specific risk assessments and safe work method statements (SWMS).

Completion of this form is mandatory for specified excavation or digging tasks (depth >300 mm), excluding movement of spoil piles/clearing drains and reclamation works within established zones where services are known not to exist and entry is not required. It must be used together with a Permit to Work and a written risk assessment.

| Section 1 – General Details |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activity Title: (As per Work Activity Risk Assessment) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scope of work: (Reason for excavation) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Likely depth of excavation work: |  |  |  | 300mm to 1.5 m |  | > 1.5 m |  |  |  |  |  |  |  |
| Section 2 – Underground Services Identification |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identification undertaken by: |  |  |  |  |  |  |  |  |  | Date: |  |  |  |
| Type of identification to be undertaken: (tick appropriate) |  | Civil or Geotechnical Engineer’s report undertaken / to be referenced |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Information sourced from DBYD AND confirmed by physical service location (e.g., vacuum excavation or electronic detection). DBYD alone is not considered sufficient.” |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Visual inspection and search of the work area and potential services in the surrounds |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Existing PBPL services maps / GIS / plans reviewed / to be reviewed |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Underground service location detection undertaken / to be undertaken |  |  |  |  |  |  |  |  |  |  |  |
| Have services been identified? |  | Yes Services have been identified that could impact on the excavation tasks. |  |  |  |  |  |  |  |  |  | Complete Section 2 |  |
|  |  | No No services have been positively identified via approved service location methods within the work area (Note: DBYD alone is not sufficient; physical service location is required.) |  |  |  |  |  |  |  |  |  | Once Evidence is supplied/ attached Proceed to Section 3 |  |
|  | Evidence attached: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Service Locator Results |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Vacuum |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Excavation Records |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Photos |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Map Mark-ups |  |  |  |  |  |  |  |  |  |  |  |
| Service type Nominate the type of service(s) identified: water pipeline, sewerage pipeline or services, stormwater pipeline or services, gas pipeline, fuel tank/pipeline, telecommunications, live/unknown electrical, non live electrical, etc: |  |  |  |  |  |  | Proximity of service (tick appropriate): |  |  |  | Depth details: (if known or as an estimation) |  |  |
|  |  |  |  |  |  |  | Service directly where excavation required | Service in proximity of required excavation |  |  |  |  |  |
| NOTE: Where a service has been identified, insert information in the service location and restrictions sections below to ensure a safe system of work. Where live electrical or pressurised services are identified within X metres (distance to be defined by SME), isolation must be undertaken before excavation begins. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Service location(s) (Provide details/description of locations &/or references to maps/plans/diagrams, testing results, paint marking of areas, etc): |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Excavation and Digging - PTW Number: |  |  |  |  |  |  |  |  |

| Barricades/hoardings ≥900 mm with ‘Danger – Do Not Enter’ signage where public or non-workers may access. Ground paint-marked: safe dig zones and service traverses. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Restrictions to ensure safe work (e.g. restrictions given by gas / fuel service owners) (Provide details of work restrictions required): |  |  |  |  |  |  |  |  |  |
| Section 3 – Excavation Collapse & Entry Controls |  |  |  |  |  |  |  |  |  |
| Is entry required? |  |  |  |  | Yes |  |  | Proceed to Section 3A |  |
|  |  |  |  |  | No |  |  | Proceed to Section 3C |  |
| 3A: | - Into an excavation greater than 1.5 m deep? |  |  |  | Yes |  |  | Proceed to Section 3B |  |
|  |  |  |  |  | No |  |  | Proceed to Section 3C |  |
| 3B: | - A safe means of entry will be achieved via (must identify one): |  |  |  | The use of secured ladders – at least one per 9m section of trench |  |  |  |  |
|  |  |  |  |  | The following alternative safe means: |  |  |  |  |
|  | - Prevention of collapse will be achieved via (must identify at least one): |  |  |  | The use of shoring |  |  |  |  |
|  |  |  |  |  | The use of battering to all sides required |  |  |  |  |
|  |  |  |  |  | The use of benching to all sides required |  |  |  |  |
|  |  |  |  |  | Groundwater conditions must be assessed prior to excavation. Where water ingress is possible, a dewatering plan must be implemented to prevent instability and collapse. |  |  |  |  |
|  |  |  |  |  | A written and signed authority (obtained from certified geo-technical engineer stating that the excavation is safe for entry) |  |  |  |  |
|  | - General entry safety will be achieved via (both items mandatory): |  |  |  | More than one person being present at the excavation during entry. Note: Working alone is prohibited in any excavation/trench >1.5 m deep. |  |  |  |  |
|  |  |  |  |  | A competent person to supervise work, inspect excavation(s) and |  |  |  |  |
|  |  |  |  |  | maintain an excavation log daily prior to entry |  |  | Proceed to Section 3D |  |
| 3C: | - General safety to be achieved via: |  |  |  | A competent person to supervise work and inspect excavation(s) |  |  |  |  |
|  | - Prevention of collapse will be achieved via (must identify at least one): |  |  |  | Collapse hazard must always be assessed. Where entry is not intended, exclusion zones, barricading, and covers for excavations are mandatory. |  |  |  |  |
|  |  |  |  |  | Shoring, battering, or benching is required for excavations deeper than 0.5m unless a geotechnical assessment confirms stability. Discuss and confirm with SMEs. Note: Procedure minimum applies shoring/battering/benching for >1.5 m or unstable soils <1.5 m.”) |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Proceed to Section 3D |
| 3D: | - Clarifying details as applicable (about type of shoring, method of placement/removal, batter/bench dimensions, access details, etc): |  |  |  | Set mobile plant, materials, and spoil ≥ 600 mm from the edge. |  |  |  |  |
|  |  |  |  |  | Use buffer boards/rails as required. |  |  |  |  |
|  |  |  |  |  | Other: |  |  |  |  |
| Section 4 – Additional Considerations |  |  |  |  |  |  |  |  |  |
| Tick as appropriate? |  |  | Close-by exhaust fumes, outputs, etc could affect the excavation atmosphere. |  |  |  | Include relevant control details within the Risk Assessment for the Work Activity |  |  |
|  |  |  | Atmospheric testing required for excavations >1.5 m or where contamination is suspected. If atmosphere cannot be assured, classify excavation as Confined Space and attach Confined Space Control Form; manage under PBPL PTW. |  |  |  |  |  |  |
|  |  |  | Exclusion / barricading is to be erected to exclude access / prevent falls |  |  |  |  |  |  |
|  |  |  | Controls are required to prevent undermining of near-by structures |  |  |  |  |  |  |
|  |  |  | Emergency rescue plan documented, including rescue equipment and trained personnel |  |  |  |  |  |  |
|  |  |  | The area is likely to contain contaminated soil / old process materials / chemicals |  |  |  |  |  |  |
|  |  |  | New services will need to be marked / identified &/or service plans updated |  |  |  |  |  |  |
|  |  |  | If applicable, include a reference to TMP and controls for vehicle interaction. |  |  |  |  |  |  |
|  |  |  | Run-off controls in place (silt fences, drain covers/sieves, signage) to prevent soil/materials entering roads/drains/creeks |  |  |  |  |  |  |

| Excavation and Digging - PTW Number: |  |
| --- | --- |

| Section 5 – Emergency Response |  |  |
| --- | --- | --- |
| Rescue Plan Reference: - Emergency Contact Number: - Site Emergency Contact: - Supervisor: - First Aider: | Immediate Actions: - Stop work immediately - Evacuate to safe area - Isolate energy sources if safe - Notify emergency services | Location of Emergency Equipment: - Fire Extinguishers: - Emergency Shower/Eye Wash: - First Aid Kits: - Other: |